Network Topology Design
In the HQ(Gauteng):

- HQ hosts core services (AD DS, DNS, (File Services)

- Redundancy implemented through firewalls and routers and backup server

- The small departments are for security and easy management

The small branches:

- Firewall for redundancy and protection

- And router to route the data out of the branch to other branches

- The switch is for interconnecting all the devices

- The Head Office (Gauteng) has the main internet connection.

- Other branches connect securely to HQ using Site-to-Site VPN/WAN links

- Branches can then access:

  - Internet

  - Servers

  - Active Directory

  - Shared files

through the HQ network

Subnetting
Class A network:10.0.0.0/8
The private network that we selected: 10.93.0.0/8.

- Calculations
Head Office  (Gauteng):
2^5 = 32 addresses
32-2 = 30 usable hosts
Network bits = 32 – 5= 27 
Subnet mask = 255.255.255.224

Other branches (Cape Town, Free State, Eastern Cape):
Initially, the branch offices were assigned /29 but the /29 subnet was found to only accommodate the 5 practical devices and it is also too small, because the branches required additional devices.

2^4 = 16 addresses
16-2 = 14 usable hosts
Network bits = 32 – 4= 28 
Subnet mask = 255.255.255.240

| Location | Network | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| Gauteng | 10.93.0.0/27 | 255.255.255.224 | 10.93.0.1 |
| Cape Town | 10.93.2.0/28 | 255.255.255.240 | 10.93.2.1 |
| Free State | 10.93.3.0/28 | 255.255.255.240 | 10.93.3.1 |
| Eastern Cape | 10.93.4.0/28 | 255.255.255.240 | 10.93.4.1 |

PRACTICAL IMPLEMENTATION — GAUTENG ONLY

Virtual Machines

- Windows Server 2019

- Windows 7

STEPS

On Windows Server

Install:

- Active Directory Domain Services

- DNS Server

Add  new forest: school.local

On Windows 7(Client PC)
 Connect the pc to server using the domain credentials